XMT®

350 Series

Issued Nov. 2019 • Index No. DC/18.93

# Multiprocess Welding

Power Source

Quick Specs

Heavy Industrial Applications

Construction Shipbuilding Railroad

Truck/trailer manufacturing Fabrication

Repair shops Rental fleets

Power generation plants

Processes

MIG (GMAW)/Pulsed MIG (GMAW-P)*

Stick (SMAW) TIG (GTAW)

Flux-cored (FCAW)

Air carbon arc cutting and gouging (CAC-A) Rated: 1/4-inch carbons

Capable: 5/16-inch carbons

*Available on select models.

Input Power Auto-Line™, 208 – 575 V,

requires 3- or 1-phase power

Output Range 10 – 38 V, 15 – 425 A

Net Weight 80 lb. (36.3 kg)

Auxiliary  Power (optional)

120 V, 10 amps

## Portability and excellent arc performance make the XMT family the most popular in the industry.

With multiprocess capabilities and features like Auto-Line, the XMT 350 is portable and versatile for applications from jobsites to the factory floor.

AUTO-LINE™

#### Input power management for portability

CONVENIENT

HANDLES

LESS SPACE

Lightweight and portable

WELDS

WITH LESS

REWORK

BETTER

XMT 350 CC/CV shown

Best multiprocess capabilities

Power source is warranted for three years, parts and labor. Original main power rectifier parts are warranted for five years.

Miller Electric Mfg. LLC

An ITW Welding Company 1635 West Spencer Street

P.O. Box 1079

Appleton, WI 54912-1079 USA

### Equipment Sales US and Canada

Phone: 866-931-9730

FAX: 800-637-2315

International Phone: 920-735-4554

International FAX: 920-735-4125

### MillerWelds.com

XMT 350 CC/CV

XMT 350 MPa

Choosing the Correct XMT®  350

MOST POPULAR!

XMT 350 CC/CV

Flexibility and simplicity make this the most popular model. It has the core multiprocess capabilities along with the flexibility of a 14-pin for spool guns, feeders, and remote controls.

XMT 350 MPa

Built-in pulse programs for manufacturing and fabrication applications that have benefits for standard steels, high-strength steels and aluminum.

| Input Power |  | Auto-Line™ 208 – 575 V, 3- or -1-phase |  |
| --- | --- | --- | --- |
| Input Voltage Limits |  | 190 – 630 V |  |
| Weld Output |  | 350 A at 34 VDC, 60% duty cycle |  |
| Carbon Arc Gouging |  | Rated: 1/4-inch carbons, Capable: 5/16-inch carbons |  |
| Net Weight |  | 80 lb. (36.3 kg) |  |
| Output Connections |  | Dinse-style |  |
| Stick/TIG/MIG |  |  |  |
| Pulsed MIG |  |  |  |
| Compatible Accessories | Insight Core™ |  |  |
|  | ArcReach® SuitCase® Feeders |  |  |
|  | Bench Feeders |  |  |
|  | Spool Guns |  |  |
|  | Remote Controls |  |  |

XMT®  350 Series Features

Versatility

For portability and reliability, Auto-Line allows for any input voltage hookup

(208 – 575 V, single- or three-phase) with no manual linking, providing convenience in any job setting. Ideal solution for dirty or unreliable input power.

14-pin receptacle has the flexibility to be used with spool guns, feeders, and remote controls.

Optional 115-volt auxiliary power provides 10 amps of circuit-breaker- protected power for coolant systems, etc.

#### Optional Welding Intelligence™ system.

XMT CC/CV and MPa models are Insight-capable to monitor weld voltage, amperage, and arc-time and percentage.

### Multiprocess weld quality

High-quality performance in all welding processes.

Adaptive Hot Start™ makes starting stick electrodes easy without creating an inclusion.

Infinite arc control available in the stick and wire modes for easier fine tuning of tough-to-weld materials and out-of-position applications.

Lift-Arc™ provides arc starting that minimizes contamination of the electrode and without the use of high-frequency.

#### XMT 350 CC/CV panel shown

Reliability

Wind Tunnel Technology.™  Internal air flow that protects electrical components and PC boards from dirt, dust and debris… greatly improving reliability.

Fan-On-Demand™ cooling system operates only when needed, reducing noise, energy use and the amount of contaminants pulled through machine.

Lightweight aerospace-grade aluminum case offers protection with the benefit of reduced weight.

Dinse-style weld disconnects provide high-quality weld cable connections.

Note: Two Dinse-style connectors are supplied with each Dinse machine.

### Welder friendly control panel

Process selector switch reduces the number of control setup combinations without reducing any features.

Large, dual digital meters are easy to view and are presettable to ease setting weld output.

Ultra-tough, polycarbonate-blended cover protects front controls from damage.

Primary voltage display shows what primary voltage the power source is linked for without removing the side panels. Saves time and assures you that the power source has been correctly linked for available input voltage.

Auto Remote Sense™ enables unit to automatically use remote control if connected to remote control receptacle.

### Portability

Jobsite mobility is made easy with a small, compact design and Auto-Line technology.

Additional XMT® 350 MPa Features

## Built-in pulse programs for manufacturing and fabrication applications that have benefits for standard steels, high-strength steels and aluminum.

#### Setup indicator lights

- Arc control indicator

- Wire type indicator

- Gas type indicator

- Setup button

Additional features when using a 70 Series MPa Plus feeder or XR-AlumaFeed® feeder.

Synergic pulsed MIG. As you increase/ decrease the wire feed speed, the pulse parameters increase/decrease, matching the right amount of power output to match the wire speed, eliminating the need to make additional adjustments.

Profile Pulse™ provides TIG appearance with MIG simplicity

and productivity. Achieve “stacked dimes” without gun manipulation. Profile Pulse frequency can be changed to increase or decrease the spacing between the ripple pattern to achieve the desired weld appearance.

Added capabilities with Insight Core.™ When using an MPa Plus feeder, wire deposition is added to the Insight Core capabilities.

Advanced arc control enables an improved pulse welding arc. The operator has more control over puddle and bead characteristics using SharpArc.. In addition, the unit provides a more robust and stable arc at very low arc length, reducing heat input, and providing the ability to weld on a wider range of material thickness.

(0 setting)

(50 setting)

SharpArc® controls the arc in pulsed MIG mode and gives total control over the arc cone shape, puddle fluidity and bead profile. If a gas is used other than what is listed in the chart above, SharpArc can be

adjusted to help customize your arc to the gas being

Low Setting	High Setting

used. Lower SharpArc settings widen the arc cone, increase puddle fluidity and flattens the weld bead.

Built-in pulsed MIG programs provide reduced heat affected zone, weld in all positions, great for thick-to-thin metal, good gap filling ability and faster travel speeds and deposition. See the table below for the wide range of pulsed MIG (GMAW-P) welding applications the XMT MPa models can handle.

| Wire Types — diameter in inches (mm) |  | Gas Types |
| --- | --- | --- |
| Steel | .035 (0.9) STL .045 (1.2) STL | ARGN CO2 (argon/carbon dioxide) 80 ARGN CO2 (argon/carbon dioxide) ARGN OXY (argon/oxygen) |
| Steel 100S | .035 (0.9) STL, .045 (1.2) STL | 100S C5 (95 argon/5 carbon dioxide) |
| Metal Core | .045 (1.2) MCOR, .052 (1.4) MCOR | ARGN CO2 (argon/carbon dioxide) |
| Stainless Steel | .035 (0.9) SSTL .045 (1.2) SSTL | TRI MIX (tri-gas mixture) ARGN OXY (argon/oxygen) ARGN CO2 (argon/carbon dioxide) |
| Aluminum | .035 (0.9) AL4X (4000 Series) .040 (1.0) AL4X (4000 Series) 3/64 (1.2) AL4X (4000 Series) 1/16 (1.6) AL4X (4000 Series) | ARGN (argon) |
|  | .035 (0.9) AL49 (4943) .040 (1.0) AL49 (4943) 3/64 (1.2) AL49 (4943) 1/16 (1.6) AL49 (4943) | ARGN (argon) |
|  | .035 (0.9) AL5X (5000 Series) .040 (1.0) AL5X (5000 Series) 3/64 (1.2) AL5X (5000 Series) 1/16 (1.6) AL5X (5000 Series) | ARGN (argon) HE AR25 (helium/argon) |
| Nickel | .035 (0.9) NI .045 (1.2) NI | ARGN HE (argon/helium) ARGN (argon) |
| Copper Nickel | .035 (0.9) CUNI, .045 (1.2) CUNI | HE ARGN (helium/argon) |
| Silicon Bronze | .035 (0.9) SIBR, .045 (1.2) SIBR | ARGN (argon) |
| Titanium | .035 (0.9) TI-5, .045 (1.2) TI-5 | ARGN HE25 (75 argon/25 helium) |
|  | .035 (0.9) TI-5, .045 (1.2) TI-5 | ARGN HE50 (50 argon/50 helium) |

Specifications (Subject to change without notice.)

| Input Power | Voltage Range in CV Mode | Amperage Range in CC Mode | Rated Output | Amps Input at Rated Load Output, 60 Hz 208 V 230 V 400 V 460 V 575 V KVA KW |  |  |  |  |  |  | Max. Open- Circuit Voltage | Dimensions | Net Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Three-phase | 10 – 38 V | 5 – 425 A | 350 A at 34 VDC, 60% duty cycle | 40.4 | 36.1 | 20.6 | 17.8 | 14.1 | 14.2 | 13.6 | 75 VDC | H: 17 in. (432 mm) W: 12.5 in. (318 mm) D: 24 in. (610 mm) | 80 lb. (36.3 kg) |
| Single-phase | 10 – 38 V | 5 – 425 A | 300 A at 32 VDC, 60% duty cycle | 60.8 | 54.6 | 29.7 | 24.5 | 19.9 | 11.7 | 11.2 |  |  |  |

Certified by Canadian Standards Association to both the Canadian and U.S. Standards.

Performance Data

#### Duty Cycle Chart

| 3-ph | ase operatio | n |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-ph | ase operatio | n |  |  |  |  |  |  |  |  |
|  |  |  |  | 6 AW | G power | cord |  |  |  |  |
|  |  |  |  | 8 AW (supp | G power lied with | cord power | source) |  |  |  |

500

425

400

350

Welding Amperes

300

250

200

150

100

10	15	20	25	30

40	50	60	70

80 90 100

% Duty Cycle

Genuine Miller®  Accessories

## Remote Controls

Wireless Remote Foot Control 300429 For remote current and contactor control.

Receiver plugs directly into the 14-pin receptacle

of Miller machine.

#### Extension Cables for Remote Controls and 24 VAC Wire Feeders

242208025 25 ft. (7.6 m)

242208050 50 ft. (15 m)

242208080 80 ft. (24.3 m)

14-pin plug to 14-pin socket. Not for use with XR-AlumaFeed® or 70 Series MPa feeder.

#### MIGRunner™ Cart

195445 For single feeders

Small footprint and easily maneuverable, with dual-cylinder rack low enough that you do not have to lift bottles. Durable ergonomic handles are designed for comfort.

Note: Not compatible with dual wire feeders.

Running Gear Cylinder Rack 300408 For single or dual feeders

Holds two large gas cylinders and has gun cable hangers and a consumable drawer in front. A convenient handle allows the cart to be pulled easily through doorways. Power source and single or dual feeders can be mounted to cart and secured.

MIG kit 300405

shown.

Industrial MIG 4/0 Kit with Dinse Connectors 300405 For single feeders

300956 For dual feeders

Consists of flowmeter regulator with 10-foot (3 m) gas hose, 10-foot (3 m) 4/0 feeder weld cable, and 15-foot (4.6 m) work cable with 600-amp C-clamp. Dual kit comes with two flowmeter regulators and gas hoses.

90-foot (27.4 m) operating range.

#### Wireless Remote

Hand Control 300430

For remote current and contactor control.

Receiver plugs directly into the 14-pin receptacle of Miller machine.

300-foot (91.4 m) operating range.

#### RFCS-14 HD Foot

Control 194744 Maximum flexibility is accomplished with a reconfigurable cord that can exit the front, back

or either side of the pedal. Foot pedal provides remote current and contactor control. Includes 20-foot (6 m) cord and 14-pin plug.

#### RHC-14 Hand Control 242211020

20-foot (6 m) cord

#### 242211100

100-foot (30.5 m) cord Miniature hand control for remote current and

contactor control. Dimensions: 4 x 4 x 3.25

inches (102 x 102 x 82 mm). Includes cord and 14-pin plug.

#### RCC-14 Remote Contactor and Current Control 151086

East/west rotary-motion fingertip control attaches to TIG torch using two hook-and-loop fasteners. Includes 26.5-foot (8 m) cord and 14-pin plug.

#### RCCS-14 Remote Contactor and Current Control 043688

North/south rotary-motion fingertip control attaches to TIG torch using two hook-and-loop fasteners. Includes 26.5-foot (8 m) cord and 14-pin plug.

## Cable Connectors and Adapters

Note: XMT 350 power sources are equipped with Dinse-style connectors for secondary connections.

#### Dinse-Style Connectors

042418 Accepts #4 to #1/0 AWG cable 042533 Accepts #1/0 to #2/0 AWG cable Kits include one Dinse-style male plug which attaches to the work and/or weld cables and plugs into the Dinse-style receptacles on the power source.

Extension Kit for Dinse-Style Cable Connectors 042419 Accepts #4 to #1/0 AWG cable.

Used to adapt or extend weld and/or work cables. Kit includes one male Dinse-style plug and one in-line female Dinse-style receptacle.

Extensions for Dinse-Style Cable Connectors 134460 Male Dinse-style plug

136600 Female Dinse-style receptacle Used to adapt or extend weld and/or work cables. Accepts #1/0 to #2/0 AWG cable.

#### Dinse/Tweco® Adapter 042465

Dinse/Cam-Lok Adapter 042466

One-piece adapter with Dinse-style male plug (to power source) on one end and Tweco or Cam-Lok female receptacle (for weld cable connection) on other end.

## Wire Feeders/Guns

ArcReach®  SuitCase® 8

951726 With Bernard® BTB Gun 300 A 951728 With Bernard® Dura-Flux™ gun ArcReach® SuitCase® 12

951729 With Bernard® BTB Gun 300 A

951731 With Bernard® Dura-Flux™ gun

Portable feeder designed to run off of arc voltage. When paired with an ArcReach power source, provides remote control of the power source without a control cord — saving time and money. See literature M/6.55.

#### SuitCase® 12RC  951580

Lightweight, portable wire feeder for a variety of wires up to .062 inch. Holds up to a 12-inch (12RC) spool of wire. Includes remote voltage control, drive roll kit and Bernard® BTB Gun 300 A. See literature M/6.5.

#### XR™-S Control 300601

Controls offer unsurpassed performance and reliability for feeding aluminum and other difficult-to-feed wire types. Models include

.035-inch and factory-installed 3/64-inch drive rolls. See literature M/1.7.

#### XR-AlumaFeed® 300509

Genuine Miller®  Accessories (Continued)

Advanced push-pull wire feeder designed for high-volume, high-production aluminum MIG welding where quality and consistency is a high priority. See literature DC/34.0.

#### 70 Series

S-74S  951196

S-74D  951198

Features Posifeed™ wire drive assembly and handles a variety of wires up to 1/8 inch.

Includes Bernard® BTB Gun 400 A. Dual-wire models also available. See literature M/3.0.

#### 70 Series MPa Plus

S-74 MPa Plus 951291 D-74 MPa Plus 951292

Same features as traditional 70 Series plus more. Handles a variety of wires up to 5/64 inch. Includes Bernard® BTB Gun 400 A. Works synergically with MPa power sources. See literature M/3.0.

Extension Cables 247831025 25 ft. (7.6 m)

247831050 50 ft. (15.2 m)

247831080 80 ft. (24.4 m)

Eleven conductors to support contactor control and remote voltage control on all Miller® electronic CV 14-pin power sources. Additional functions supported when using MPa power sources include synergic pulsed MIG, remote process select and side select capabilities.

#### Spoolmatic® 30A 130831

WC-24 (137549) required for use with 14-pin receptacles, order separately. A one-pound, air-cooled spool gun that extends the weld process range by adding a handy portable system for small wire MIG. Rated 200 amps at 100% duty cycle. Includes 30-foot (9.1 m) cable assembly.

#### WC-24 Weld Control 137549

Required to use Spoolmatic 30A spool gun. Designed for use with Miller CV power sources with 14-pin receptacles and supplying 24 VAC.

## Welding Intelligence™

#### Insight Core™ 14-pin Module 301072

Easy-to-install module brings Insight Core capabilities to your existing fleet. This simplified, Internet-based weld data solution collects, transmits and presents actionable information to any Web-connected device. Insight Core can help you increase productivity, improve weld quality and manage costs.

Ordering Information Most popular models

Power Source and Options	Stock No.	Description	Qty.	Price

| XMT® 350 CC/CV | 907161 907161011 907161012 | 208 – 575 V with Auto-Line™ 208 – 575 V with Auto-Line™ and auxiliary power 230 – 460 V with Auto-Line™ and auxiliary power and CE |  |  |
| --- | --- | --- | --- | --- |
| XMT® 350 CC/CV MIGRunner™ Packages | 951786 951314 | Power source 907161 with 20 Series Basic feeder Power source 907161 with S-74D feeder Packages also include MIGRunner cart, Bernard ® BTB Gun, drive roll kit, 10 ft. 2/0 work cable with clamp, 6 ft. 2/0 wire feeder to power source weld cable, and flowmeter regulator with 3 ft. gas hose |  |  |
| XMT® 350 MPa | 907366 907366011 907366002 907366014 | 208 – 575 V with Auto-Line™ 208 – 575 V with Auto-Line™ and auxiliary power 230 – 460 V with Auto-Line™ and auxiliary power and CE 208 – 575 V with Auto-Line™ and Tweco® connectors |  |  |
| Gas Valve Kit | 195286 | For XMT 350 |  |  |
| Accessories |  |  |  |  |
| MIGRunner™ Cart | 195445 | For single feeders |  |  |
| Running Gear Cylinder Rack | 300408 | For dual feeders |  |  |
| Industrial MIG 4/0 Kit with Dinse Connectors | 300405 300956 | For single feeders For dual feeders |  |  |
| Cable Connectors and Adapters |  | See page 6 |  |  |
| Remote Controls |  |  |  |  |
| Wireless Remote Foot Control | 300429 | Foot control with wireless 90 ft. (27.4 m) operating range |  |  |
| Wireless Remote Hand Control | 300430 | Hand control with wireless 300 ft. (91.4 m) operating range |  |  |
| RFCS-14 HD | 194744 | Foot control with 20 ft. (6 m) cord and 14-pin plug |  |  |
| RHC-14 | 242211020 242211100 | Hand control with 20 ft. (6 m) cord Hand control with 100 ft. (30.5 m) cord |  |  |
| RCC-14 | 151086 | Fingertip control |  |  |
| RCCS-14 | 043688 | Fingertip control |  |  |
| Extension Cables (8-conductor) |  | See page 6 |  |  |
| Wire Feeders/Guns |  |  |  |  |
| ArcReach® SuitCase® 8 | 951726 951728 | With Bernard® BTB Gun 300 A. See literature M/6.55 With Bernard® Dura-Flux™ gun |  |  |
| ArcReach® SuitCase® 12 | 951729 951731 | With Bernard® BTB Gun 300 A. See literature M/6.55 With Bernard® Dura-Flux™ gun |  |  |
| SuitCase® 12RC | 951580 | With Bernard® BTB Gun 300 A. See literature M/6.5 |  |  |
| XR™-S Control | 300601 | Standard model with wire feed speed digital display, jog/purge, run-in control, and trigger hold. See literature M/1.7 |  |  |
| XR-AlumaFeed® | 300509 | Advanced model adds Profile Pulse™, synergic control, trigger schedule select, lock capabilities and flow meter. See literature DC/34.0 |  |  |
| 70 Series | 951196 951198 | S-74S. See literature M/3.0 S-74D. See literature M/3.0 |  |  |
| 70 Series MPa Plus | 951291 951292 | S-74 MPa (single-wire). Works synergically with MPa power sources D-74 MPa (dual-wire). Works synergically with MPa power sources |  |  |
| Extension Cables (11-conductor) |  | See page 7 |  |  |
| Spoolmatic® 30A Spool Gun | 130831 | Requires WC-24 for use with 14-pin receptacles |  |  |
| WC-24 Weld Control | 137549 | Required for use with Spoolmatic® 30A and power sources with 14-pin |  |  |
| Bernard® MIG Guns |  | See Bernard literature SP-BTB |  |  |
| Welding Intelligence |  | See page 7 |  |  |

Date:	Total Quoted Price:

Distributed by:

© 2019 Miller Electric Mfg. LLC